Embedded Systems Design

20153208  Lee soyoung

PROJECT : SMART_MIRROR

What is Smart Mirror??

- A smart mirror that shows you what you want using a mirror as a display monitor

The Motivation of proposal

- Difficult to manage time while watching the mirror

What you want to implement

- Mirror is to display monitor

- The mirror show that time, date , weather ,etc…..

- We can control the mirror through voice recognition.

- Time reminder and smart phone status display via mirror

Actual production

1.Hardware Making

- Necessary article : , LCD monitor , Clear glass , Mirror film,Wood, Black tape,5mm Non-woven,tool etc….

- Attach half mirror film of same size on transparent glass of 45 * 55 size

- Remove the air bubbles between the mirror film and the transparent glass by pushing them down.

- Prepared wood is cut into 57 * 2.5 * 2.5 size and 44 * 2.5 * 2.5 size and make four each

- Use wood to make two rectangular frames.

- Put the glass with the mirror film between two frames to nail it.

- Fill the space created by the mirror with nonwoven fabric and nail on the edge of the mirror.

- Remove the bezel of the monitor with a flat screwdriver.

- Cover the bezel part of the monitor and the back of the mirror with black tape.

Image

2.Software production

- Necessary article : Raspberry Pi ,LCD monitor , Speaker, Usb microphone etc…

2-1.Raspberry Pi Set

- Raspbian jessie downloads and OS seting

- Sudo rapi-config : lanquage and reqional set

- Sudo passwd root : passwd set

- Wifi set : iwconfig -> iwlist wlan0 scan -> su - -> wpa_passphrase wifi_name wifi_pw >> /etc/wpa_supplicant/wpa_supplicant.conf -> Su pi -> sudo ifup wlan0

- Korean-fonts set : sudo apt-get install ttf-unfonts-core -> sudo apt-get install ibus-hangul

- Speaker set : sudo raspi-config -> advanced Options -> 1.Force 3.5mm jack -> finish -> you can test aplay test.wav

- Microphone set : sudo vi usr/share/alsa/alsa.conf -> defaults.ctl.card 1,daefaults.pcm.card 1

- Display rotation set : sudo vi/boot/config.txt -> (bottom line) displayrotate=1

- Other setting url reference : (http://jeongchul.tistory.com/425?category=552705)

2-2. Smart Mirror Display

- Node download and set : wget https://nodejs.org/dist/v.4.0.0-linux-armv7l.tar.gz -> tar -xvf this file

- Github Opensource load : make dictory -> git clone https://github.com/evancohen/smart-mirror.git

- Node-module load : npm install

- Test-smart-mirror : cp main.js -> npm start

- Customizing open source css, html, and js using Eclipse tools

2-3. Voice recognition.

- Attempt to implement speech recognition using nodejs

- Attempt to implement speech recognition using python

- Failed to implement : I am sorry that the period is short.